Technical Specifications

Purchase of GPS tracking device

The Contracting Authority defines by these technical conditions the basic characteristics of the subject-matter of the Public Contract, i.e. the minimum technical parameters that the equipment must meet

The equipment offered must not be inferior in any of the parameters.

The Contracting Authority requires the delivery of new equipment.

The Supplier shall add the manufacturer and type of equipment offered: /to be completed by the Supplier/

| Minimum technical specifications: | Offered by Supplier YES / NO and for numerical requirements, the Supplier shall also add the exact value of the device they are offering |
| --- | --- |
| weight max. 6 g |  |
| antenna length max. 10 cm |  |
| operating temperature min -10°C - 50°C |  |
| battery life - min. 100 GPS fixes without recharging |  |
| Rechargeable with solar panels |  |
| GSM global roaming support |  |
| Average GPS accuracy (CEP (50%)) &lt;= 10 m |  |
| Waterproof and dustproof - min degree of protection IP 68 |  |
| possibility of automatic firmware updates - OTA |  |
| Other available sensors: minimum accelerometer (provided min. ODBA), thermometer and luxmeter |  |
| The possibility of data backup (if there is no signal to send), with sufficient capacity to collect and store data for at least 1 year |  |
| Ability to interrupt data collection when the device is not installed on the animal |  |
| Option to export data to Movebank - archiving, viewing and continuous downloading of data from the provider's server |  |
| Data services (data transmission, positioning) |  |