Technical Specification

Public procurement: Wood Processing Control Simulator for Sawmill

The contracting authority defines the basic characteristics of the requested subject of the contract with these technical conditions, i.e. the minimum technical parameters that the offered goods must meet.

The offered goods must not be inferior in any of the parameters.

The contracting authority requires the delivery of new, non-refurbished, and unused goods.

The participant in the tender procedure must complete all the yellow-highlighted fields in the tables below.

| Wood Processing Control Simulator for Sawmill |  |
| --- | --- |
| Specific manufacturer and type of the offered goods: |  |
|  | The supplier will fill in the parameters of the offered device. |
| General requirements for the device: |  |
| The device must be designed for effective and safe training of industrial sawmill operators. |  |
| Technical requirements: |  |
| A faithful simulation of real working conditions – sawmill environment. |  |
| The ability to train various operations (at least log orientation, cutting speed, response to incidents, frame saw cutting, band saw cutting, and aggregate cutting). |  |
| Real-time feedback to track student progress, results monitoring on the teacher's PC. |  |
| Configuration of different types of wood (hardwoods, softwoods) and peripheral control elements. |  |
| The ability to track and analyze student performance – statistics on yield and productivity on the teacher's PC. |  |
| The ability to customize training plans according to the individual needs of students. |  |
| Student account management: Ability to manage student accounts and learning paths.. |  |
| Educational modules: Available educational modules for different skill levels (at least 3 levels - beginner, intermediate, and advanced). |  |
| Languages: At least English. |  |
| Accessories – description and technical specifications: |  |
| Floor area: max. 2.5 m² |  |
| 2 screens: HD screen min. 46" and touch screen min. 15". |  |
| Control elements: controlled via joysticks and pedals. |  |
| Connectivity: min. Wi-Fi and LAN connection. |  |